Using this template

This guide is designed to help your organization build a written safety program.

You’ll need to customize the template to fit your organization—it’s not ready to use as is. We’ve made that process easier by adding visual prompts throughout the document. Look for yellow-highlighted, red text that shows where you need to add or update information.

You can (and should) change any part of the template to match your organization’s structure, such as department names, job titles, responsibilities, or procedures.

Example

Before:

<COMPANY NAME>

Sample Safety Program

After:

XYZ Company

XYZ Safety Program

Formatting tips

- To remove the yellow highlighting: Select the highlighted text, then click the highlighter icon in the Font menu.

- To change the red font to black: Select the text and click the font color icon.

You’ll also see “Check your understanding” boxes throughout the template. These help you think through how to customize certain sections. After you’ve read and completed each one, simply right-click the box and select “Cut.”

Disclaimer. This sample safety program template cannot be used as is. You must customize the template to meet the needs of your organization. EMC does not guarantee that this template is or can be relied on for compliance with any law or regulation, assurance against preventable losses, or freedom from legal liability. We make no representations or warranties of any kind whatsoever, either express or implied, in connection with the use of this template. EMC will not be liable for your use of the template as customized by you. All safety programs and policies, including this template and the information you supply to complete it, should be reviewed by your legal counsel and/or risk management staff.

<Company Name>

Forklift Safety Program

Purpose and Scope

<Company Name> is committed to providing a safe and healthy work environment and to protecting our employees from injury or death caused by uncontrolled hazards in the workplace. The Forklift Safety Program has been established to reduce the risk of physical injury or property damage in areas where powered forklifts and other powered material handling equipment is in operation.

The Forklift Safety Program applies to all employees (permanent, temporary, and contractors) who operate forklifts and other powered material handling equipment at our facilities.

Program Responsibilities

Management. <Company Name> is responsible for providing safe equipment and the resources necessary to implement this program, and for ensuring that this program is being followed by the Program Administrator.

Program Administrator. The Program Administrator is <Name>. They are responsible for developing and implementing our Forklift Safety Program, including:

- Annually reviewing and updating this written plan

- Providing appropriate training on the safe operation of all powered forklift equipment used within

<Company Name> facilities

- Documenting all training and evaluations

- Observing forklift operations and reporting unsafe practices to the appropriate supervisor

- Reviewing copies of inspection checklists

Supervisors. Supervisors are responsible for:

- Ensuring that employees who are found to have insufficient skills or understanding of safe forklift

operations receive retraining before continuing to operate a forklift

- Ensuring employees comply with all safe work practices described in this program

- Observing forklift operations in their department and correcting any unsafe practices

- Providing program feedback to the Program Administrator

Forklift Operators. Operators are responsible for:

- Only operating the forklifts for which they have been specifically trained and authorized

- Operating all forklifts in a safe manner, consistent with the forklift safe work practices

- Conducting forklift inspections at the beginning of each work shift and documenting the inspection

on the appropriate inspection forms

- Reporting all equipment malfunctions and/or maintenance needs to their supervisors immediately

- Wearing a seatbelt at all times while operating a forklift

- Notifying their supervisors if they begin taking medication that affects their ability to operate a

forklift

Pre-Qualification

All forklift operator candidates must meet the following basic requirements prior to starting initial or refresher training:

- No uncorrectable vision problems that would impair the safe operation of a forklift

- No uncorrectable hearing loss that would impair the safe operation of a forklift

- No physical limitations that would impair the safe operation of a forklift

- No neurological disorders that affect balance or consciousness

- No medication that affects perception, vision or balance

Initial Employee Training

Operator training occurs before an employee is permitted to operate any forklift or other powered material handling equipment in our facilities. All operational training will be conducted under close supervision. Training will consist of a combination of formal instruction (e.g., lecture, discussion, video), practical training (demonstrations performed by the trainer and exercises performed by the trainee), and an evaluation of the operator’s performance in the workplace.

Only knowledgeable and experienced forklift operators that are authorized by <Company Name> are permitted to conduct training and evaluations. The Program Administrator has authorized the following people to provide forklift training.

| Names of Authorized Employees |  |
| --- | --- |
| 1. | <Name> |
| 2. | <Name> |
| 3. | <Name> |

All training and evaluations are documented and will include the name of the trainee, name of the trainer and the date of training. All training will be recorded using the form in Appendix A.

Training Program Content

Training for forklift operators is extensive, and covers both general forklift topics and workplace-specific topics, including:

General forklift topics:

- An overview of <Company Name>’s written Forklift Safety Program

- Operating instructions, warnings and precautions for the types of forklifts the operator will be

authorized to operate

- Differences between forklifts and automobiles

- Forklift controls and instruments

- Engine or motor operation

- Steering and maneuvering

- Visibility (including restrictions due to loading)

- Fork and attachment adaptation, operation and use limitations

- Vehicle capacity and stability

- Vehicle inspection and maintenance

- Refueling and/or charging of batteries

- Seatbelt use

Workplace-specific topics:

- Surface conditions where the vehicle can be operated

- Load stability, manipulation, stacking and unstacking

- Pedestrian traffic areas

- Narrow aisles and other restricted places

- Hazardous locations where the vehicle will be operated

- Ramps and other sloped surfaces that could affect the vehicle’s stability

- Closed environments and other areas where a buildup of carbon monoxide or diesel exhaust could

exist

- Any other unique or potentially hazardous environmental conditions in the workplace that could

affect safe operation

Refresher Training

Refresher training will be conducted to ensure that all operators have the knowledge and skills needed to operate forklifts safely. Refresher training will be conducted for individual operators when the operator:

- The operator has received an evaluation that reveals he/she is not operating the forklift safely

- The operator has been observed operating the vehicle in an unsafe manner

- The operator has been involved in an accident or near-miss incident

- The operator is assigned to drive a different type of forklift

- A condition in the workplace has changed which could affect the safe operation of the forklift

Refresher training will be evaluated by the Program Administrator to assess the effectiveness of the training.

Operator Evaluation

Each forklift operator’s performance is evaluated every three years. This evaluation includes a discussion with the operator regarding his/her experience with the forklift, an observation of the employee operating the forklift, and written documentation that the evaluation was performed. All evaluations will be documented on the form located in Appendix B. Individuals that do not pass the evaluation will be immediately removed from forklift operations until they successfully pass refresher training.

Forklift Safe Work Practices

The following procedures will be followed at all times.

Equipment Inspection.

- Each shift the forklift operator will inspect the forklift before operation. If any inspection item is determined to be damaged, broken or inoperable, the operator will notify their supervisor and authorized mechanic. If possible, the forklift will be immediately repaired. If immediate repair is not possible, a determination will be made as to whether the forklift can be used safely until repairs can be made.

- Employees may not operate an unsafe forklift at any time.

- Forklifts will be kept in clean condition, free of dirt, excess oil and grease.

Repairs and Maintenance.

- Only <Company Name> authorized mechanic will perform repairs and maintenance on forklifts and other powered material handling equipment.

- The authorized mechanic will complete a maintenance log that identifies repair needs and corrective actions taken for each forklift. This log is kept at the <Location>

- If a forklift cannot be safely operated, it must be taken out of service until the repairs have been made. Forklifts that have been taken out of service will be visually marked with an out of service sign and the ignition keys will be secured in the <Location>.

- After repairs have been completed, the forklift must be given a performance test before being returned to service to ensure that the equipment is safe to operate.

Changing and Charging Batteries.

- Forklift batteries will only be changed and charged in the designated areas.

| Designated Area Locations: |  |
| --- | --- |
| 1. | <Location> |
| 2. | <Location> |
| 3. | <Location> |

- Equipment is provided at battery-charging areas to safely flush and neutralize spilled battery acid and electrolytes.

- Smoking is prohibited in all battery-charging areas.

- Eyewash equipment is available and maintained in all charging areas.

- Precautions will be taken to prevent open flames, sparks and electric arcs in charging areas.

- Employees who change and service batteries and handle corrosive liquids must wear PPE including:

- Long sleeve shirts and full-length pants

- Face shields

- Safety glasses or goggles

- Chemical aprons

- Chemical gloves

- The following steps must be taken when charging forklift batteries.

- Set emergency brake

- Remove the battery cover

- Check for plugged vent caps

- Turn off the battery charger then connect it to the battery

- Avoid touching battery terminals with any metal object as this could cause a spark

- Once batteries are charged turn off the battery charger

- Disconnect the battery charger from the battery and replace the battery cover

Changing LP Gas Cylinders.

- Forklift LP cylinders will only be changed in the designated areas.

| Designated Area Locations: |  |
| --- | --- |
| 1. | <Location> |
| 2. | <Location> |
| 3. | <Location> |

- All full and empty cylinders will be kept in the appropriately labeled cylinder cage when not on a forklift.

- Smoking is prohibited in all cylinder changing areas.

- Eyewash equipment is available and maintained in all changing areas.

- Precautions are taken to prevent open flames, sparks and electric arcs in changing areas.

- Employees who change cylinders must wear appropriate PPE including:

- Long sleeve shirts and full-length pants

- Face shields

- Safety glasses or goggles

- Gloves

- The following steps must be taken when changing all forklift LP cylinders.

- Set emergency brake and leave the engine running

- Close tank valve and use up remaining fuel in the fuel lines

- Turn off ignition

- Disconnect the hose from the tank and remove the tank

- Inspect the new tank for leaks and damage (do not use if there is a leak or dents in the tank)

- Install the new tank and connect the hose

- Open the tank valve and restart the engine

General Safe Work Practices.

- Only authorized and trained personnel are permitted to operate forklifts.

- Horseplay is prohibited.

- Operators must drive with both hands on the steering wheel. Do not drive with wet or greasy hands.

- No person is permitted to ride as a passenger on a forklift or on the load being carried.

- A forklift may not be used to elevate a platform or pallet with people on it, except work platforms specifically designed for this purpose. Work platforms must have standard guardrails and must be securely fastened to the forks.

- No person is allowed to stand or walk under elevated forks.

- Operators should avoid jerky starts, quick turns or sudden stops.

- Operators may not use the reverse gear as a brake.

- Operators must slow down when on wet and slippery surfaces and at cross aisles or locations where vision is obstructed.

- Operators entering a building or nearing a blind corner must make their approach at a reduced speed, sound their horn and proceed carefully.

- Operators must give pedestrians right-of-way at all times.

- Operators may not drive toward any person who is in front of a fixed object or wall.

- Operators may not overtake and pass another forklift traveling in the same direction.

- Operators must not put their fingers, arms or legs between the uprights of the mast, or beyond the contour of the forklift.

- Forks should always be placed under the load as far as possible. Do not lift a load with one fork.

- No load should be moved unless it is absolutely safe and secure.

- Spotters must be used when handling long lengths of bar stock, pipe or other materials.

- Compressed gas cylinders may be moved only in special pallets designed for this purpose.

- When unloading trucks or trailers, the brakes on the vehicle must be set (locked) and the wheels chocked.

- Forklifts must be safely parked when not in use. The controls must be neutralized, power shut off, brakes set, key removed, and the forks must be in a down position, flat on the surface, and not obstructing any walkways or aisles.

- A forklift will not be left on an incline unless it is safely parked and the wheels chocked.

- Only stable and safely arranged loads may be handled.

- Only loads within the rated capacity of the forklift may be lifted or moved.

Traveling.

- Facility speed limits must be observed at all times.

- Three forklift lengths (or two seconds) must be maintained between forklifts in operation.

- The forklift must be kept under control at all times.

- When vision is obscured, the operator must slow down and sound the horn.

- If the load blocks the operator’s view, the forklift must be driven in the direction that provides the best visibility.

- Forklifts must cross railroad tracks at a diagonal.

- Forklifts must be parked 8 feet or further from the center line of railroad tracks.

- The forklift must be driven with the load upgrade when driving on ascending or descending grades greater than 10%.

- Dock boards and bridge plates must be properly secured before they are driven over.

- When the forklift is not carrying a load, the operator must travel with the forks as low as possible (maximum of 3 inches on paved surfaces). When carrying a load, it should be carried as low as possible (consistent with safe operation, 2 to 6 inches above the surface.)

- The forks may not be operated while the forklift is traveling.

Periodic Program Review

The Forklift Safety Program and procedures are reviewed annually. The review is documented on the form located in Appendix C.

Revision History

Revision 1 - <Date>

Appendix A – Forklift Operator Training Record

The following individuals received training on <Company Name>’s Forklift Safety Program.

| Print Name | Sign Name |
| --- | --- |

The undersigned conducted training in accordance with <Company Name>’s Forklift Safety Program.

| Print Instructor’s Name |  |
| --- | --- |
| Instructor’s Signature |  |
| Instructor’s Title |  |
| Date of Training |  |
| Equipment Type |  |

Appendix B – Forklift Operator’s Evaluation

Forklift Operator’s Name ________________________________________________ Forklift Model/# ______________

Location _______________________________________________________________  Date _____________________

Evaluator’s Name __________________________________________________________________________________

| Pre-trip | Acceptable | Not Acceptable |
| --- | --- | --- |
| Conducts pre-shift inspections per operating manual and uses form/checklist |  |  |
| Looks for damage and reports problems |  |  |
| Elevators | Acceptable | Not Acceptable |
| Does not exceed elevator capacity, centers forklift, drives in load first and allows no other passengers |  |  |
| Elevating Personnel | Acceptable | Not Acceptable |
| Only elevates personnel using approved work platform |  |  |
| Properly secures work platform to forklift |  |  |
| Does not travel with personnel in platform and checks raising and lowering of mast and platform before raising personnel |  |  |
| Ensures that forklift has twice the capacity of the platform weight and load (including personnel) |  |  |
| Barricades area |  |  |
| Stays at forklift controls while personnel are elevated |  |  |
| Traveling | Acceptable | Not Acceptable |
| Wears safety belt or harness and required PPE |  |  |
| Keeps body within operator compartment |  |  |
| Operates forklift in accordance with operating manual instructions |  |  |
| Looks in direction of travel before and while moving |  |  |
| Uses forklift lighting in dark areas |  |  |
| Carries forks/load 3" to 6" above ground |  |  |
| Smooth starts, stops and direction changes |  |  |
| Sounds horn at blind corners - slows or stops as necessary |  |  |
| Turns wide to see down travel path |  |  |
| Travels right of center allowing room for step out |  |  |
| Uses extreme caution when meeting pedestrians |  |  |
| Leaves three or more lengths for stopping when behind another vehicle |  |  |
| Stays well away from drop-offs |  |  |
| Load Handling | Acceptable | Not Acceptable |
| Never handles loads in excess of forklift capacity or load tiers above LBR height |  |  |
| Uses attachments according to manufacturer’s instructions |  |  |
| Approaches load properly |  |  |
| Does not raise or lower forks while traveling |  |  |
| Does not turn with forks elevated |  |  |
| Keeps mast vertical (load level) when high stacking |  |  |
| Enters and exits pallets properly (forks level and properly spaced) |  |  |
| Travels in reverse when load partially obstructs visibility |  |  |
| Travels with loads 6" - 12" above the ground and tilted back to stabilize the load |  |  |
| Handles long loads with forks spread wide and uses spotters |  |  |
| Aligns loads properly in racks or stacks |  |  |
| Pedestrian Issues | Acceptable | Not Acceptable |
| Slows, honks and yields to pedestrians at corners and step outs |  |  |
| Allows no pedestrians near operating forklift particularly in trailers or railcars |  |  |
| Watches for pedestrians in tail swing area |  |  |
| Honks before passing pedestrians |  |  |
| Allows no pedestrians beneath the load or to ride on forklift, forks or pallet |  |  |
| Trailer and Railcar Loading | Acceptable | Not Acceptable |
| Trailer/railcar properly chocked; jack stand used when needed |  |  |
| Slides axles to rear |  |  |
| Inspects floor |  |  |
| Checks dock board capacity; installs correctly |  |  |
| Takes steps to prevent tractor-trailer pull-away |  |  |
| Uses lighting aids |  |  |
| Looks into trailer/railcar before entering with each load |  |  |
| Ramps/Inclines | Acceptable | Not Acceptable |
| Travels with load upgrade and takes precautions for visibility |  |  |
| Travels empty with counterweight upgrade |  |  |
| Never turns on ramps or inclines |  |  |
| Parking | Acceptable | Not Acceptable |
| Lowers forks, sets brake, neutralizes transmission controls, chocks wheels on slope and shuts off LPG when parking for extended periods of time |  |  |
| Parks in locations not blocking firefighting equipment, electrical panels, doorways, stairways, ladders, emergency exits or railroad tracks |  |  |
| Shuts off engine if moving more than 25 feet from forklift or moving out of sight of forklift |  |  |
| LPG/CNG | Acceptable | Not Acceptable |
| Turns off forklift |  |  |
| Allows no open flames or ignition sources in refueling area |  |  |
| Wears required PPE |  |  |
| LPG: Removes tank and stands bottle vertically to refuel |  |  |
| If authorized, fills in accordance with regulations |  |  |
| Uses outage valve to determine fill level and does not overfill |  |  |
| Seeks assistance replacing bottle on forklift and uses locator pin and hole to properly index bottle |  |  |
| Cleans up spills |  |  |
| Battery Charging | Acceptable | Not Acceptable |
| Turns off forklift |  |  |
| Allows no open flames or ignition sources in refueling area |  |  |
| Wears required PPE |  |  |
| Opens battery lid - does not remove caps |  |  |
| Does not plug running charger into battery |  |  |
| Battery Changing | Acceptable | Not Acceptable |
| Turns off forklift |  |  |
| Allows no open flames or ignition sources in refueling area |  |  |
| Wears required PPE |  |  |
| Locates forklift properly, uses changing device in accordance with manufacturer’s instructions, and securely reinstalls battery and retention device |  |  |
| Additional Observations and Comments |  |  |

Appendix C – Annual Evaluation Report

| Date of Evaluation: | Evaluated By (list all present): |
| --- | --- |
| Written Program Reviewed: Yes No |  |
| Comments on Written Program: |  |
| The following specific procedures have been reviewed: |  |
| The following specific procedures were modified: |  |
| The following specific procedures were added: |  |
| A review of the accident reports and injury and illness reports were made: Yes No |  |
| The following additional expense(s) resulted from failure to properly operate a forklift or other powered material handling equipment: |  |
| Comments: |  |